| Assessed By: |  | Kevin Lovett |  |  |  |  |  |  |  |  | Risk Assessment No: |  |  |  |  | RA03 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hazard Identified: |  | Plastering |  |  |  |  |  |  |  |  | Date Reviewed: |  |  |  |  | June 2025 |  |
|  |  |  | A |  |  |  |  | B |  |  |  | C |  |  |  | D |  |
|  |  |  | Who’s at Risk? |  |  |  |  | Severity |  |  |  | Likelihood |  |  |  | Risk Rating |  |
|  |  |  | E |  | C | P |  | 3 | 2 | 1 |  | 3 | 2 |  | 1 | #### B x C = D |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 |  |
| Risk Assessment Ratings | 6-9 High Risk |  |  |  |  |  | 4 Medium Risk |  |  |  |  |  |  | 1-3 Low Risk |  |  |  |
| Key |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A E = Employees C = Contractors P = Public / 3rd parties |  |  |  | B 3 = Death or Major injury 2 = Reportable injury 1 = Minor Injury – Time off unlikely |  |  |  |  |  |  |  | C 3 = Very likely 2 = Possible 1 = Unlikely or Very Unlikely |  |  |  |  |  |
| Hazard Control Measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Residual Risk |
| - Only trained and competent operatives to be employed for plastering operations; - Be aware if there are other site operatives in your works area due to the use of the honed trowel. If there is a risk to others barrier off your work area; - Clear routes of access and egress are to be maintained during the works; - Where working at height is required suitable equipment for the task is to be selected; - Mix plaster close to the area where it is required to reduce manual handling once the mix is made i.e. use a hose so that water is close to the work area; - When mixing use a mixing drill and mask to eliminate manual handling and prevent the inhalation of dust; - Ensure that competent plasterers are using correct manual handling techniques; - When working in a building with an unknown structure you should request the asbestos register; - Competent persons are to isolate and remove electrical switches and fittings prior to the plastering being undertaken; - Use a wheel barrow or pallet truck to move bags of plaster in order to eliminate manual handling; - Ensure that suitable PPE is worn this shall include safety gloves and PPE which has been defined by site rules. When mixing use suitable RPE (FFP-3 minimum) to avoid dust exposure; - Use gloves to maintain good standards of hygiene and to keep hands protected and clean; - Request hazard data sheets for the products to be used and carry out a COSHH assessment. Refer to the product specific requirements for its use, storage, PPE/RPE, etc.; - Ensure any waste is segregated accordingly and regularly removed to prevent a build-up which may cause a safety hazard; - Store materials safely to avoid damage and accidental displacement; - Clear up any spills immediately so that a slip hazard is not presented to yourself or others; - If an electrical mixer is used for plaster ensure that vents remain clear, leads are not damaged and clothing etc. cannot become entangled in the drill mechanism; - Drills will be PAT tested every 3 months; - Electrical cables are to be routed away from aisles and walkways where they could create trip hazards to other persons and/or expose cables to potential damage. Where possible support cables at height so that the hazard is eliminated; - When plastering ceilings eye protection is to be worn; - When dry sweeping add some water to the ground to keep the dust from becoming airborne; - Open the windows and doors to reduce the dust within the working environment but be considerate of third parties who may be affected. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Low (2) |