| Assessed By: |  | Kevin Lovett |  |  |  |  |  |  |  |  | Risk Assessment No: |  |  |  |  | RA10 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hazard Identified: |  | Use of hop-ups |  |  |  |  |  |  |  |  | Date Reviewed: |  |  |  |  | June 2025 |  |
|  |  |  | A |  |  |  |  | B |  |  |  | C |  |  |  | D |  |
|  |  |  | Who’s at Risk? |  |  |  |  | Severity |  |  |  | Likelihood |  |  |  | Risk Rating |  |
|  |  |  | E |  | C | P |  | 3 | 2 | 1 |  | 3 | 2 |  | 1 | #### B x C = D |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6 |  |
| Risk Assessment Ratings | 6-9 High Risk |  |  |  |  |  | 4 Medium Risk |  |  |  |  |  |  | 1-3 Low Risk |  |  |  |
| Key |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A E = Employees C = Contractors P = Public / 3rd parties |  |  |  | B 3 = Death or Major injury 2 = Reportable injury 1 = Minor Injury – Time off unlikely |  |  |  |  |  |  |  | C 3 = Very likely 2 = Possible 1 = Unlikely or Very Unlikely |  |  |  |  |  |
| Hazard Control Measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Residual Risk |
| - Only to be used on site at the Supervisor / Foreman discretion and when all other forms of access equipment have been considered; - Only proprietary Hop-Ups are to be used not crates or carpenters horses etc.; - Operatives shall be given training in the assembly, storage and safe use of Hop-Ups. The training shall be recorded and kept in the form of a register / tool box talk; - All Hop-Ups shall be checked for defects by a competent individual before each use. Any defects must be reported to a supervisor; - A competent person shall inspect all Hop-Ups every week. The result of each of the inspections shall be recorded in the inspection register; - Any defective equipment shall be taken out of use until repairs have been undertaken or the equipment is replaced; - Working platforms should be of a minimum of 600mm x 600mm, unless there is sufficient justification for a smaller width; - Operatives are to follow manufacturer’s instructions for the use and storage of the equipment; - Only one person must use the Hop-Up at any time; - Before use ensure that the Hop-Ups are on firm, flat, stable ground and the area is clear of debris; - Hop-Ups must be regularly cleaned to minimise the accumulation of plaster or any other material; - Boxes / crates / insulation board or other means must not be used to gain extra height from the Hop-Up working platform. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Low (3) |